ALU-PLAN®	tender specifications	type ES 10 / 90°

pos. ______	deliverty and assembly of 2-part stainless-steel corner beads

deliverty	type ES 10 / 90° with angle-armatures

material / view side:	 stainless-steel well polished 240

with protective foil

strength (in mm):	 1,5  2,0  3,0

radius (in mm):	 r=2xstrength

thigh lengths (in mm):	 40x40  50x50  60x60

lengths (height):	________ mm

amount / exemplars:	________ exemplars

unit price:	________ exemplars / Euro

summary price			____________ Euro

fabricator / proof of delivery:

ALU-PLAN® GmbH

Hohe Straße 7	e-mail: info@alu-plan.de

D-92245 Kümmersbruck	internet: http://www.alu-plan.de

Germany	phone: +49 (0)9624 / 92 234 345

or equivalent according

pos. ______	installation of the preposition aluminium corner beads

installation	on double-made plasterboard stud wall

insert all angle-armatures in a distance of 60 cm into the profile before

starting the installation. in the first step the profile has to mount on the

prime gypsum cardboard with the angle-amatures by screws. pay

attention, that the profil is fixed exactly horizontal and vertical without

tolerance. use a laser-level for this step. all unevennesses could be

compensated with a gypsum-kit.

the additional wall surfaces like wall tiles can be fixed

after the installation of the corner beads

further the construction management should order profile-protections

(for example foil fixings) against the paint- or plastering works

further the construction management order the remove of all profile-

protections (for example foil fixings)

unit price:	________ exemplars

summary price			____________ Euro