ALU-PLAN®	tender specifications	type ES 12 / 90°

pos. ______	deliverty and assembly of aluminium corner beads

deliverty	type ES 12-A / 90°

material / view side:	 aluminium anodized E6 / EV1

base profil:	 aluminium pressed

strength (in mm):	 2,0

radius (in mm):	 25,0

thigh lengths (in mm):	 55x55

lengths (height):	________ mm

amount / exemplars:	________ exemplars

unit price:	________ exemplars / Euro

summary price			____________ Euro

fabricator / proof of delivery:

ALU-PLAN® GmbH

Hohe Straße 7	e-mail: info@alu-plan.de

D-92245 Kümmersbruck	internet: http://www.alu-plan.de

Germany	phone: +49 (0)9624 / 92 234 345

or equivalent according

pos. ______	deliverty and assembly of aluminium corner beads

alternativ	how preposition with a powder-coating type ES 12-F / 90°

on RAL-Card K1 RAL ________ (without metallic-colors)

in metallic-color  RAL 9006  RAL 9007

in DB- color  701  702  703

___ exemplars	exemplars / €	________	€	________

pos. ______	installation of the preposition aluminium corner beads

installation	on double-made plasterboard stud wall

in the first step the base-profile has to mount on the prime

gypsum cardboard by screws. pay attention, that the profil

is fixed exactly horizontal and vertical without tolerance.

use a laser-level for this step. pay attention, that the upper-

profile concludes exactly with the wall surface of the second

gypsum cardboard. all unevennesses could be compensated

with a gypsum-kit.

the base-profile must be cleaned up after the installation

for a better cleaning you should wipe the profil-bridges of the

base-profil with a free acid oil before the installation

further the construction management should order

profile-protections (for example foil fixings) against

the paint- or plastering works

the installation of the upper-profiles have to start after the

electronic-, paint- and floor-works. further the construction

management should determine the time of installation for

the upper-profiles

further the construction management order the remove

of all profile-protections (for example foil fixings)

unit price:	________ exemplars

summary price			____________ Euro