ALU-PLAN®	tender specifications	type ES 20 / 90°

pos. ______	deliverty and assembly of aluminium corner beads

deliverty	type ES 20-A / 90°

material / view side:	 aluminium anodized E6 / EV1

strength (in mm):	 2,0

radius (in mm):	 2,0

thigh lengths (in mm):	 50x50

lengths (height):	________ mm

amount / exemplars:	________ exemplars

unit price:	________ exemplars / Euro

summary price			____________ Euro

fabricator / proof of delivery:

ALU-PLAN® GmbH

Hohe Straße 7	e-mail: info@alu-plan.de

D-92245 Kümmersbruck	internet: http://www.alu-plan.de

Germany	phone: +49 (0)9624 / 92 234 345

or equivalent according

pos. ______	deliverty and assembly of aluminium corner beads

alternativ	how preposition with a powder-coating type ES 20-F / 90°

on RAL-Card K1 RAL ________ (without metallic-colors)

in metallic-color  RAL 9006  RAL 9007

in DB- color  701  702  703

___ exemplars	exemplars / €	________	€	________

pos. ______	installation of the preposition aluminium corner beads

installation	on double-made plasterboard stud wall

in the first step the base-profile has to mount on the prime

gypsum cardboard in the lateral grooves by screws. pay

attention, that the profil is fixed exactly horizontal and vertical

without tolerance. use a laser-level for this step. pay attention,

that the upper-profile concludes exactly with the wall surface

of the second gypsum cardboard. all unevennesses could be

compensated with a gypsum-kit.

perhaps the base-profile could stuck on the wall with a polymer

S78 adhesive or an equivalent adhesive material.

further, the construction management should order

profile-protections (for example foil fixings) against

the paint- or plastering works

further the construction management order the remove

of all profile-protections (for example foil fixings)

unit price:	________ exemplars

summary price			____________ Euro