ALU-PLAN®	tender specifications	type ES 25 / 90°

pos. ______	deliverty and assembly of stainless-steel corner beads

deliverty	type ES 25 / 90°

material / view side:	 stainless-steel well polished 240

thigh lengths (in mm):	 27x27 mm

lengths (height):	________ mm

amount / exemplars:	________ exemplars

unit price:	________ exemplars / Euro

summary price			____________ Euro

fabricator / proof of delivery:

ALU-PLAN® GmbH

Hohe Straße 7	e-mail: info@alu-plan.de

D-92245 Kümmersbruck	internet: http://www.alu-plan.de

Germany	phone: +49 (0)9624 / 92 234 345

or equivalent according

pos. ______	installation of the preposition aluminium corner beads

installation	on double-made plasterboard stud wall

in the first step the base-profile has to mount on the prime

gypsum cardboard by screws. pay attention, that the profil is

fixed exactly horizontal and vertical without tolerance. use a

laser-level for this step. all unevennesses could be compensated

with a gypsum-kit.

chamfer the second gypsum cardboards and adapt it to the

mounted corner bead

further, the construction management should order

profile-protections (for example foil fixings) against

the paint- or plastering works

further the construction management order the remove

of all profile-protections (for example foil fixings)

unit price:	________ exemplars

summary price			____________ Euro