ALU-PLAN®	Ausschreibungsunterlage	Typ ES 4.1 / 90°

pos. ______	deliverty and assembly of stainless-steel corner beads

deliverty	type ES 4.1 / 90°

material / view side:	 stainless-steel well polished 240

stainless-steel brushed 320

with protective foil

strength (in mm):	 1,5  2,0  3,0

radius (in mm):	 r=2xstrength

r=10mm+strength

r=25mm+strength

chamfer:	 20° on 10,0 mm

thigh lengths (in mm):	 40x40  50x50  60x60

lengths (height):	________ mm

amount / exemplars:	________ exemplars

unit price:	________ exemplars / Euro

summary price			____________ Euro

fabricator / proof of delivery:

ALU-PLAN® GmbH

Hohe Straße 7	e-mail: info@alu-plan.de

D-92245 Kümmersbruck	internet: http://www.alu-plan.de

Germany	phone: +49 (0)9624 / 92 234 345

or equivalent according

pos. ______	installation of the preposition stainless-steel corner beads

installation	on the wall

we recommend that the corner beads are stuck on,

for example with a polymer S78 adhesive

or an equivalent adhesive material.

further the construction management should order

profile-protections (for example foil fixings) against

the paint- or plastering works

further the construction management order the remove

of all profile-protections (for example foil fixings)

unit price:	________ exemplars

summary price			____________ Euro