ALU-PLAN®	tender specifications	baseboards

i series / stainless-steel

page 1 of 3

delivery      assembly      installation	please delete unimportant informations

deliverty of

pos. ______	stainless-steel baseboards

type ________

material:	 stainless-steel well polished 240

stainless-steel brushed 320

with protective foil

strength (in mm):	 1,0  1,25  1,5  2,0  3,0

width (in mm):	________ mm

height (in mm):	________ mm

lengths:	________ lfm

lfm / Euro	__,__	___,__	Euro

fabricator / proof of delivery:

ALU-PLAN® GmbH

Hohe Straße 7	e-mail: info@alu-plan.de

D-92245 Kümmersbruck	internet: http://www.alu-plan.de

Germany	phone: +49 (0)9624 / 92 234 345

or equivalent according

ALU-PLAN®	tender specifications	baseboards

i series / stainless-steel

page 2 of 3

delivery      assembly      installation	please delete unimportant informations

manufacture the inside-corners – on stump	E_MI_S01

pos. ______	all inside-corners have to be mounted on stump

during the installation all visible lips have to be deburred cleanly.

___	exemplars	exemplars / Euro	__,__	___,__	Euro

manufacture the outside-corners – on stump	E_MA_S01

pos. ______	all outside-corners have to be mounted on stump

during the installation all visible lips have to be deburred cleanly.

___	exemplars	exemplars / Euro	__,__	___,__	Euro

manufacture the outside-corners – 90° degree bending	E_MA_90

pos. ______	all outside-corners have to be bent with a bender on 90° degrees.

(possible only up to 1.5 mm strength)

___	exemplars	exemplars / Euro	__,__	___,__	Euro

manufacture a permanent elastic joining	DEV

pos. ______	during the installation the lateral grooves have to be filled

with acryl in the color of the wall.

___	lfm	lfm / Euro	__,__	___,__	Euro

ALU-PLAN®	tender specifications	baseboards

i series / stainless-steel

page 3 of 3

delivery      assembly      installation	please delete unimportant informations

installation of the stainless-steel baseboards of the pre-position	E_M_01

pos. ______	the profiles have to be glued on the wall

with a polymer S78 adhesive or an equivalent adhesive material.

all unevennesses have to be adjusted. therefore use a gypsum

setting binder for gypsum boards.

the installation must take place on an exact meter tear on all walls,

for example by using a laser control unit.

further, the construction management should order

profile-protections (for example foil fixings) against

the paint- or plastering works.

all protectives have to be removed only on arrangement

by the construction management.

___	lfm	lfm / Euro	__,__	___,__	Euro